# ESP32 Firmware Feature List

# 1. Account & Pairing

- Register/Login/Guest mode (app side)

- Pair device via WiFi (AP portal or SmartConfig)

- Pair device via BLE (alt method)

- Expose MAC + UUID/QR for app to register device

- Support adding multiple devices

# 2. LED States

- Not connected to WiFi → LED off / Red power LED

- Connecting to WiFi → Blue blink every 1s

- WiFi connected, cloud registration pending → blink continues

- WiFi + cloud confirmed (UUID/QR/MAC sent, ack received) → Blue solid ON

- Cloud/MQTT disconnected after setup → distinct pulse pattern

- Config portal active → distinct pattern

# 3. Ownership Flow (AP Portal)

- Boot unconfigured → Red LED, start SoftAP (SSID: Smart Device, PWD: fixed)

- Serve config portal at device IP

- Scan + list WiFi networks

- Accept 2 WiFi credentials, connect to strongest / fallback

- Device QR select (dropdown or camera scan)

- User info fields: name, email, mobile (validated)

- Data send interval selector

- Submit → close portal → connect WiFi → blink pattern

- Save WiFi + user config to flash

- Send to cloud via MQTT QoS2

- On cloud ack → LED solid ON

- Start publishing dummy sensor data

- Subscribe command topic → GPIO control

- Birth / Last Will MQTT messages

# 4. On-Demand Portal

- Reopen on button press(drd – on demand config)

- Don't wipe existing config on reopen

# 5. Reset

- Soft reset: clear WiFi creds only

- Hard reset: erase all user-configured flash data

# 6. MQTT

- Confirm broker version: 3.1.1 or 5.0

- Support QoS 0/1/2

- Use QoS2 for provisioning/config messages

- TLS enabled, cert handling on ESP32

- Birth message on connect

- Last Will message on disconnect

# 7. Sensor Data (Dummy Stage)

- Generate 5 random values: int, double, string

- Publish on configured interval

- JSON payload with fixed key names

# 8. Actuator Control

- Subscribe command topic

- GPIO HIGH on "1", LOW on "0"

- Publish back actuation confirmation

# 9. FOTA

- Run in separate task/thread

- Use ESP32 OTA partitioning

- Signed firmware, rollback safe

- Report firmware version as attribute

# 10. Edge / SmartConfig Variant

- App scans WiFi, sends creds via SmartConfig

- App sends WiFi + user config to Firebase

- ESP32 pulls config from Firebase (REST/HTTPS)

- Push telemetry/state to Firebase

- Add/remove sensor: dynamic config, no reflash

- Add/remove actuator: dynamic GPIO mapping

# 11. Backend Alignment

- Match MQTT payload keys to DB schema fields

- Finalize topic/payload contract before coding