E-IoT Platform

Live Demo Runbook

# 1. Overview

This runbook demonstrates every implemented section of the E-IoT Platform firmware live, using only a serial monitor and MQTT command-line tools — no dashboard or app required. Two terminal windows act as your "dashboard":

- Window 1 — Serial monitor (pio device monitor): the narrative — boot, pairing, LED state, health stats.

- Window 2 — mosquitto_sub -v on devices/<uuid>/#: the live feed of everything the device publishes.

Optional: run a third window to issue curl/mosquitto_pub commands, and/or open MQTT Explorer alongside Window 2 for a visual tree view of the same data.

## 1.1 What Each Act Proves

| Act | Sections | Proves |
| --- | --- | --- |
| 1 | 1, 2 | Fresh-device identity, WiFi/BLE pairing, LED feedback |
| 2 | 3, 6 | Cloud registration, birth/LWT, QoS2, ack-driven confirmation |
| 3 | 7, 8 | Live telemetry + remote actuator control |
| 4 | 4, 5 | Reopen-without-wiping, soft/hard reset |
| 5 | 10 | Live OTA download, verify, flash, reboot |
| 6 | — | Health-check monitoring, running the whole time |

Core runtime: ~15 minutes (Acts 1–6). Add ~5 minutes for the optional Section 11 extension.

# 2. Pre-Demo Checklist

- Flash the device once beforehand and confirm it boots clean — don't debug live.

- config.h fully filled in: real broker host, real FOTA_BASE_URL — no TODO placeholders.

- Physical actuator wired: LED + resistor on GPIO_ACTUATOR_PIN, visible to the audience.

- Broker reachable from the venue WiFi — test this in advance; some public/office WiFi blocks MQTT ports. Have a mobile hotspot as backup.

- FOTA test server prepped and running (Act 5) — metadata.json with a bumped version + correct SHA-256, binary served over HTTP.

- mosquitto_pub/mosquitto_sub installed and already tested once today.

- Two terminal windows arranged side by side (or a split pane).

- Device hard-reset to factory-fresh right before the demo, so Act 1's pairing is real.

- Optional but recommended: screen-record a full dry run as a fallback if live WiFi/broker misbehaves.

# 3. Terminal Layout

### Window 1 — Serial Monitor

pio device monitor

### Window 2 — MQTT Live Feed

Get the device UUID from Act 1's first curl call, then:

mosquitto_sub -h <broker> -t "devices/<uuid>/#" -v

### Window 3 (optional) — Command Window

Where you type the curl / mosquitto_pub commands below, so Windows 1 and 2 stay dedicated to output.

# 4. Demo Script

ACT 1 — Fresh Device — Identity, Pairing, LED Sections 1, 2 • ~3 min

- Join the "Smart Device" WiFi network from your laptop.

curl http://192.168.4.1/info

Narrate the mac/uuid/model/fw JSON — "this is what a provisioning app would show as a QR code." Copy the uuid for every command below.

curl http://192.168.4.1/networks

curl -X POST http://192.168.4.1/pair -H "Content-Type: application/json" -d "{\"ssid1\":\"<YourWiFi>\",\"pass1\":\"<password>\"}"

Point at the LED changing pattern and the serial log line confirming the connection.

ACT 2 — Ownership Flow + MQTT Sections 3, 6 • ~3 min

mosquitto_pub -h <broker> -t "devices/<uuid>/register/ack" -m "ok"

Narrate: "this stands in for the backend confirming registration." Point at the LED going solid and the serial log's "Cloud ack received" line.

ACT 3 — Live Telemetry + Actuator Control Sections 7, 8 • ~3 min

mosquitto_pub -h <broker> -t "devices/<uuid>/cmd" -m "1"

The physical LED/relay turns on; the ack appears in Window 2 a moment later. This is usually the most visually convincing moment — let it land.

mosquitto_pub -h <broker> -t "devices/<uuid>/cmd" -m "0"

ACT 4 — On-Demand Portal + Reset Sections 4, 5 • ~2 min

Do:

- Short-press the button live — portal reopens, LED changes. Narrate: "this doesn't erase anything saved."

Optional (do this LAST if at all — it disconnects the device):

- Long-press for a soft reset, or hold longer for a hard reset — device reboots unconfigured. Only do this if you're ending on Act 4, since re-pairing takes you back to Act 1.

ACT 5 — FOTA Update Sections 10 • ~4 min

mosquitto_pub -h <broker> -t "devices/all/update" -m "check"

Switch to Window 1 — narrate the live download/verify/flash log as it happens, then the reboot. After reboot the device reconnects automatically using saved config; point out the new version reported on the attributes topic.

ACT 6 — Health Check (running throughout) Sections — • ~30 sec

Direct attention to Window 1's periodic [Health] heap/stack lines that have been scrolling in the background since boot — "this has been monitoring memory the entire time."

# 5. Optional Extension — Edge/SmartConfig Variant (Section 11)

A separate build (USE_EDGE_SMARTCONFIG_VARIANT=1) — cannot run in the same boot as Acts 1–6. Demo this as its own short segment if there's time/interest, using a second flashed device or a rebuild.

- Pair via a SmartConfig-capable app instead of the SoftAP/BLE portal.

curl "https://identitytoolkit.googleapis.com/v1/accounts:signInWithPassword?key=<API_KEY>" \

-d '{"email":"<EMAIL>","password":"<PASSWORD>","returnSecureToken":true}'

curl -X PUT "https://<db>.firebaseio.com/devices/<uuid>/config.json?auth=<idToken>" \

-d '{"intervalMs":5000,"sensors":[{"id":"s1","pin":34,"type":"analog"}]}'

curl "https://<db>.firebaseio.com/devices/<uuid>/telemetry.json?auth=<idToken>"

# 6. Troubleshooting Quick Reference

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| curl: could not connect | Not joined to SoftAP, or portal already closed | Rejoin "Smart Device"; short-press button to reopen |
| mosquitto_sub shows nothing | Wrong broker/uuid, or MQTT_BROKER_HOST still a placeholder | Double-check config.h and the topic string |
| Windows curl quoting errors | PowerShell/cmd don't parse single quotes like bash | Use double quotes, escape inner quotes with \" |
| MQTT commands time out at venue | Venue WiFi blocks the broker port | Switch to mobile hotspot backup |

# 7. Full Command Cheat Sheet

Every command from this runbook, in order, for quick copy-paste during the live demo.

### Act 1

curl http://192.168.4.1/info

curl http://192.168.4.1/networks

curl -X POST http://192.168.4.1/pair -H "Content-Type: application/json" -d "{\"ssid1\":\"<YourWiFi>\",\"pass1\":\"<password>\"}"

### Act 2

mosquitto_pub -h <broker> -t "devices/<uuid>/register/ack" -m "ok"

### Act 3

mosquitto_pub -h <broker> -t "devices/<uuid>/cmd" -m "1"

mosquitto_pub -h <broker> -t "devices/<uuid>/cmd" -m "0"

### Act 5

mosquitto_pub -h <broker> -t "devices/all/update" -m "check"

### Window 2 (run before Act 1, update <uuid> once known)

mosquitto_sub -h <broker> -t "devices/<uuid>/#" -v

# 8. Post-Demo Cleanup

- Hard-reset the device (very long button press) to leave it factory-fresh for the next demo.

- Stop the local FOTA test server (Ctrl+C).

- Close Window 2's mosquitto_sub session.